CONCORDIA UNIVERSITY

Department of Electrical and Computer Engineering

COEN  6501: DIGITAL SYSTEMS AND SYNTHESIS,

Examiner: Asim Al-Khalili

Date: Monday11th, December 2017       Time allowed 3 hrs  17:45--20:45

ALL QUESTION HAVE EQUAL VALUES,              answer all questions

Notes: 1. University approved  calculators are  permitted.

2. One two sided crib sheet is permitted

===============================================================

##### Question 1

- Realize the following functions using  2:1 multiplexers.

F = A’ + B.C.D + B.C’.D’ + A.B’                   G = A’ + B’

- Implement F, 1)Using only 3-variable tables.

2) Using only 2 variable tables

3) mix of the two types.

Which one is faster? Which one has less area? Show your implementation for each table type.

##### Question 2

Design a 3-stage Left /Right Shift register starting from a state table or a state diagram.

Discuss the impact of the clock skew (delay between clock and its complement) on the operation of the shift register when using Maste Slave Flip Flop.

##### Question 3

Design a circuit to calculate  -5A.

A is a five  bit signed number, A4 A3 A2 A1 A0 .  Use BOOTH Method,

##### Question 4

- Give the steps involved in Floating Point Addition.

- Perform the  Floating Point addition of two 8 bit word operands A+B.

A=  2.25  and B= -17.75, where each of  the operands are represented by the following floating Point arrangement

Sign bit:   1 bit

Exponent: 3bits biased

Mantessa: 4 bits

##### Question 5

The sequencer circuit shown in Figure 1 uses components with intrinsic and incremental delays listed in Table 1.

a.  Identify the critical path of the circuit. Assume TCS1 and TCS2 = 0 ns

b. Calculate the maximum speed at typical operating temperature taking into

consideration fan-out and estimated interconnect delays.

Assume TCS1 and TCS2 = 0 ns.

- For TCS = 0.7 ns and TCS2 = -1.5 ns, calculate the maximum speed of operation for an ambient temperature of 70°C and power dissipation of 1 W

Assume: 1. The temperature degrading coefficient M =2

2. Each fan out has a length of 0.25 mm.

3. Thermal resistance of the package is 36°C/W.

TCS2

Z

X

CLK

TCS1

J   Q

K

D  Q

D  Q

D  Q

CLK

Figure

Table 1- Typical Propagation Delays

| Component | Tp (ns) | Input Loading (UL)* | K1 (ns/UL) | K2 (ns/mm) |
| --- | --- | --- | --- | --- |
| Inverter | 0.15 | 1 | 0.1 | 0.04 |
| NAND/NOR (2-input) | 0.24 | 2 | 0.05 | 0.2 |
| NAND (3-input) | 0.4 | 1.5 | 0.12 | 0.35 |
| Flip-Flops (CK to Q) | 1.5 | 2 | 0.1 | 0.3 |

The flip-flops are positive edge triggered with tsu = 1.5 ns and th = 0.5 ns

##### Question 6

- Write a VHDL code for a 2-1 MUX at a structural level, using std_logic.

- Draw the circuit corresponding to the following VHDL code and describe what it does.

library ieee;

use ieee.std_logic_1164.all;

entity HA_nBit is

generic (nBit: integer);

port (A: in   Std_Logic_Vector(nBit-1 downto 0);

Cin:  in   Std_Logic;

Sum: out  Std_Logic_Vector(nBit-1 downto 0);

Cout: out  Std_Logic);

End HA_nBit;

architecture Structure of HA_nBit is

component HalfAdder

port ( A, B: in  Std_Logic; Sum, Cout: out   Std_Logic);

end component;

signal C: Std_Logic_Vector(nBit-1 downto 0);

begin

HA_Generator:

for i in 0 to nBit-1 generate

LSB:

If (i=0) generate

HA: HalfAdder port map (A(i), Cin, Sum(i),  C(0));

end generate;

MSB:

If (i=nBit-1) generate

HA: HalfAdder port map (a(i), C(i-1), Cout);

end generate;

Other bits:

If (0<i and i< nBit-1) generate

HA: HalfAdder port map (A(i), C(i-1), Sum(i), C(i));

end generate;

end generate;

end structure;